UNIVERSITY OF ELECTRONICS SCIENCE AND TECHNOLOGY OF CHINA

COURSE TITTLE: -      	          DSP

SUBMMITED TO: -		 Prof. Jingran Lin

NAME:-	            	            Asif Rashid

STUDENT  ID: -			  201424010107

SCHOOL: -	Information and Communication ENGINEERING

COUNTRY: -			PAKISTAN

DATE:				  2014-12-18

Design of FIR Filters

- FIR as a class of LTI Filters

Transfer function of the filter is

Finite Impulse Response (FIR) Filters:

N = 0, no feedback

Let us consider an FIR filter of length M (order N=M-1, watch out! order – number of delays)

Can immediately obtain the impulse response, with x(n)= (n)

The impulse response is of finite length M, as required

The FIR filters have only zeros (no poles). Hence known also as all-zero filters FIR filters also known as feed-forward or non-recursive, or transversal. Digital FIR filters cannot be derived from analog filters – rational analog filters cannot have a finite impulse response.

Why bother?

1. They are inherently stable

2. They can be designed to have a linear phase

3. There is a great flexibility in shaping their magnitude response

4. They are easy and convenient to implement .They implement FFT in a very fast manner

- FIR Filter using the DFT

FIR filter:

Now N-point DFT (Y(k)) and then N-point IDFT (y(n)) can be used to compute standard convolution product and thus to perform linear filtering (given how efficient FFT is)

Linear-phase filters

The ability to have an exactly linear phase response is one of the most important of FIR filters

A general FIR filter does not have a linear phase response but this property is satisfied when

for linear phase filter types

Linear-phase filters – Filter types

Some observations:

• Type 1 – most versatile

• Type 2 – frequency response is always 0 at w=pi – not suitable as a high-pass

• Type 3 and 4 – introduce a pi/2 phase shift, frequency response is always 0 at w=0- – not suitable as a  high-pass

- FIR Design Methods

• Impulse response truncation – the simplest design method, has undesirable frequency domain characteristics, not very useful but intro to …

• Windowing design method – simple and convenient but not optimal, i.e. order achieved is not minimum possible

• Optimal filter design methods

Approximated filters obtained by truncation

Window Design Method

To be expected …

Truncation is just pre-multiplication by a rectangular window

This is not very clever – obviously one introduces a delay

spectrum convolution

Rectangular Window Frequency Response

Magnitude of Rectangular Window Frequency Response

Truncated Filter

Ideal Requirements

Ideally we would like to have

•  small – few computations

• close to a delta Dirac mass for

our ideal low-pass filter

These two requirements are conflicting!

Increasing the dimension of the window

The width of the main lobe decreases as M increases. The area under side lob remains constant as M increases.

Conflicting Ideal Requirements

Solution to Sharp Discontinuity of Rectangular Window

Use windows with no abrupt discontinuity in their timedomain response and consequently low side-lobes in their frequency response. In this case, the reduced ripple comes at the expense of a wider transition region but this However, this can be compensated for by increasing the length of the filter.

Alternative Windows –Time Domain

Many alternatives have been proposed, e.g.

• Hanning

• Hamming

• Blackman

Windows –Magnitude of Frequency Response

Summary of Windows Characteristics

We see clearly that a wider transition region (wider main-lobe) is compensated by much lower side-lobes and thus less ripples.

Filter realised with rectangular/Hanning windows

M=16

There are much less ripples for the Hanning window but that the transition width has increase.

Filter realised with Hanning windows

Transition width can be improved by increasing the size of the Hanning window to M = 40

Windows characteristics

• Fundamental trade-off between main-lobe width and side-lobe amplitude

• As window smoother, peak side-lobe decreases, but the main-lobe width increases.

• Need to increase window length to achieve same transition bandwidth.

Specification necessary for Window Design Method

wc - cutoff frequency

&- maximum passband

ripple

change in &m– transition bandwidth

&m – width of the window

Mainlobe

Response must not enter shaded regions

Key Properties

- Passband / stopband ripples

Passband / stopband ripples are often expressed in dB:

passband ripple = 20 log10 (1+&p ) dB,

or peak-to-peak passband ripple = 20 log10 (1+&p) dB;

minimum stopband attenuation = -20 log10 &s ) dB.

Summary of Window Design Procedure

• Ideal frequency response has infinite impulse response

• To be implemented in practice it has to be truncated shifted to the right (to make is causal)

• Truncation is just pre-multiplication by a rectangular window the filter of a large order has a narrow transition band  however, sharp discontinuity results in side-lobe interference independent of the filter’s order and shape Gibbs phenomenon

• Windows with no abrupt discontinuity can be used to reduce Gibbs oscillations (e.g. Hanning, Hamming, Blackman)

Summary of the Key Properties of the Window Design Method

1. Equal transition bandwidth on both sides of the ideal cutoff frequency.

2. Equal peak approximation error in the pass-band and stopband.

3. Distance between approximation error peaks is approximately equal to the width of the window main-lobe.

4. The width of the main-lobe is wider than the transition band.

5. Peak approximation error is determined by the window shape, independent of the filter order.

Summary of the windowed FIR filter design Procedure

1. Select a suitable window function

2. Specify an ideal response Hd()

3. Compute the coefficients of the ideal filter hd(n)

4. Multiply the ideal coefficients by the window function to give the filter coefficients

5. Evaluate the frequency response of the resulting filter and iterate if necessary (typically, it means increase M if the constraints you have been given have not been satisfied) Step by Step Windowed Filter

Design a type I low-pass filter according to the specification

Step 1. Select a suitable window function. Choosing a suitable window function can be done with the aid of published data such as

The required peak error spec &2 = 0.01, i.e. -20log10 &s ) = - 40 dB Hanning window

Main-lobe width ws- wp = 0.3pi-0.2pi=0.1pi i.e. 0.1pi=8pi/M : filter length M ≥ 80, filter order N ≥ 79

Type-I filter have even order N = 80

although for Hanning window first and last ones are 0 so only 78 in reality

Step 2 Specify the Ideal Response

Property 1: The band-edge frequency of the ideal response

if the midpoint between ws and wp

wc = (ws + wp)/2 = (0.2pi+0.3pi)/2 =0.25pi

our ideal low-pass filter frequency response

Step 3 Compute the coefficients of the ideal filter

• The ideal filter coefficients hd are given by the Inverse Discrete time Fourier transform of Hd(w)

• Delayed impulse response (to make it causal)

• Coefficients of the ideal filter

• it will be computed approximately using an N-point Inverse Fast Fourier Transform (IFFT).

• Given a value of N (choice discussed later), create a sampled version of Hd ():

Hd(p) = Hd(2(pi)p/N), p=0,1,...N-1.

[frequency spacing 2/N rad/sample ]

If the Inverse FFT, and hence the filter coefficients, are to be purely realvalued, the frequency response must be conjugate symmetric:

Hd(-2(pi)p/N) = Hd*(2(pi)p/N)         (1)

Since the Discrete Fourier Spectrum is also periodic, we see that

Hd(-2(pi)p/N) =Hd(2(pi)- 2(pi)p/N)=Hd(2pi(N-p)/N)   (2)

Equating (1) & (2) we must set Hd(N-p) = Hd * (p) for p = 1, ..., (N/2-1).

The Inverse FFT of Hd *(p) is an N-sample time domain function h´(n). For h´(n) to be an accurate approximation of h(n), N must be made large enough to avoid time-domain aliasing of h(n), as illustrated below.

Time domain aliasing

Consider FFT and IFFT

The relationship (2) provides the reconstruction of the periodic signal xn however it does not imply that we can recover xn from the samples. For sequence xn of finite duration L this is only possible if N ≥ L

Step 4 Multiply to obtain the filter coefficients

• Coefficients of the ideal filter

• Multiplied by a Hamming window function

Step 5 Evaluate the Frequency Response and Iterate

The frequency response is computed as the DFT of the filter coefficient vector. If the resulting filter does not meet the specifications, one of the following could be done

• adjust the ideal filter frequency response (for example, move the band edge) and repeat from step 2

• adjust the filter length and repeat from step 4

• change the window (and filter length) and repeat from step 4

Multi-band Design

Frequency sampling method

FIR Filter Design Using Windows

FIR filter design based on windows is simple and robust, however, it is not optimal:

• The resulting pass-band and stop-band parameters are equal even though often the specification is more

strict in the stop band than in the pass band unnecessary high accuracy in the pass band

• The ripple of the window is not uniform (decays as we

move away from discontinuity points according to side-lobe pattern of the window) by allowing more freedom in the ripple behavior we may be able to reduce filter’s order and hence its complexity

FIR Design by Optimisation: Least-Square Method We now present a method that approximates the desired frequency response by a linear-phase FIR amplitude function according to the following optimality criterion.

The integral of the weighted square frequency-domain error is given by

e^2=∫E2(w)dw

and we assume that the order and the type of the filter are known. Under this assumptions designing

the FIR filter now reduces to determining the coefficients that would minimise e^2

Design a type I low-pass filter according to specification

The filter designed using window method cannot benefit from this relaxation, however, a least-square method design gives N = 33 (compared to N = 80)

Least-Square Design of FIR Filters

• Meeting the specification is not guaranteed a-priori, trial and error is often required. It might be useful to

set the transition bands slightly narrower than needed, and it is often necessary to experiment with

the weights

• Occasionally the resulting frequency response may be peculiar. Again, changing the weights would help

to resolve the problem

Equiripple Design

The least-square criterion of minimizing  2 = ∫E2()d is not entirely satisfactory. A better approach is to minimize the maximum error at each band = max|E()|

The method is optimal in a sense of minimising the maximum magnitude of the ripple in all bands of interest, the filter order is fixed

It can be shown that this leads to an equiripple filter – a filter which amplitude response oscillates uniformly between the tolerance bounds of each band

Remez method

• There exists a computational procedure known as the Remez method to solve this mathematical

optimization problem.

• There are also exist formulae for estimating the required filter length in the case of lowpass,

bandpass and narrow transition bandwidths. However, these formulae are not always reliable so it

might be necessary to iterate the procedure so as to satisfy the design constraints.

Equiripple Design: Weights

The weights can be determined in advance from a minimax specification. For example, if a simple lowpass filter has a requirement for the passband gain to be in the range 1-∂p to 1+∂p, and the stopband gain to be less than ∂s, the weightings given to the passband and stopband errors would be ∂s

and ∂p respectively. The detailed algorithm is beyond the (time!) constraints of this module.

The Parks-McClellan Remez exchange algorithm

The computational procedure the optimization problem is by Remez The algorithm in common use is by Parks and McClellan. The Parks-McClellan Remez exchange algorithm is widely available and versatile. Important: it designs linear phase (symmetric) filters or antisymmetric filters of any of the standard types.

Linear Symmetric Filters

The frequency response of the direct form FIR filter may be rearranged by grouping the terms involving the first and last coefficients, the second and next to last, etc.

and then taking out a common factor exp( -jM(omega)/2):

If the filter length M+1 is odd, then the final term in curly brackets above is the single term bM/2, that is the centre coefficient ('tap') of the filter.

Symmetric impulse response:

If we put bM = b0, bM-1 = b1, etc., and note that exp(J(omega)) +exp(-j) = 2cos(omega), the frequency response becomes

This is a purely real function (sum of cosines) multiplied by a linear phase term, hence the response has linear phase, corresponding to a pure delay of M/2 samples, ie half the filter length.

A similar argument can be used to simplify antisymmetric impulse responses in terms of a sum of sine functions (such filters do not give a pure delay, although the phase still has a linear form pi/2-m(omega)/2)

Limitations of the algorithm

Linear phase in the stopbands is never a real requirement, and in some applications strictly linear phase in the passband is not needed either.

The linear phase filters designed by this method are therefore longer than optimum non-linear phase filters.

However, symmetric FIR filters of length N can be implemented using the folded

delay line structure

Further options for FIR filter design

More general non-linear optimisation (least squared error or minimax) can of course be used to design linear or non-linear phase FIR filters to meet more general frequency and/or time domain requirements.